Hands-on Practice for MMCLAB

Shijie Yan

2019-08-08

Finish the following tasks step by step:

- Load the given brain mesh “3-0Months”, plot the cross-section plane where x+y=150 mm.

- Set the optical properties to different media types of the mesh according to http://mcx.space/wiki/index.cgi?MMC/Colin27AtlasMesh

- Add a uniform disk source with r = 10 mm at [70, 75, 160] mm. The illumination source points towards

–z direction, but is divergent with a focus distance of 20 mm.

- Add a planar detection plane of size 20×20 mm2. The plane is parallel to x-z plane and the center of the detection plane is [140, 75, 80] mm.

- Merge the illumination plane and detection plane with the skull model, and have the modified mesh ready for simulation.

- The total simulation time should be 5 ns and each time gate width is 0.1 ns.

- Choose photon counts based on your computer (suggested: 3e6-1e7). Use a random seed, consider reflection index mismatch. Save detected photons with exit information as well as their seeds.

- Run MMCLAB with your defined parameters, record the time of your simulation

- Plot the photon fluence distribution (Continuous Wave) on ‘y = 75’ mm plane (in log10 scale). Remember to use the mesh before re-tessellation. Can you also plot contours?

- In a different figure, plot the Temporal Spread Function (TPSF) of the detected photons.

- Is the number of detected photons large enough? If not, try using more photons or increasing the size of the detection plane. Reducing the scattering values might also help.

- Let’s try building the absorption Jacobian (Continuous Wave) when you are satisfied with the detected photon counts. Again, plot the Jacobian on ‘y = 75’ mm plane.